MAISSER-PE — revised and new tables (generated by make_tables.py)

Table 2. Evaporation rates and forcing for the fifteen experimental windows.

| Window | Date | Cover | Cov. (%) | Time | E (mm h⁻¹) [95 % CI] | Rs (W m⁻²) | u (m s⁻¹) | Tw (°C) | Ta (°C) | ea (kPa) | VPD (kPa) | Δe (kPa) | E/VPD | E/Δe |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W01 | 17 Aug | none | 0.0 | 09:20–17:26 | 0.349 [0.308, 0.390] | 106 | 1.94 | 26.8 | 30.7 | 2.24 | 2.16 | 1.28 | 0.161 | 0.272 |
| W02 | 18 Aug | V1 | 77.0 | 11:20–18:20 | 0.101 [0.076, 0.127] | 144 | 1.93 | 27.4 | 30.5 | 2.55 | 1.82 | 1.10 | 0.056 | 0.092 |
| W03 | 19 Aug | none | 0.0 | 10:20–15:41 | 0.476 [0.442, 0.511] | 213 | 2.16 | 29.0 | 32.1 | 2.36 | 2.43 | 1.66 | 0.196 | 0.288 |
| W04 | 20 Aug | none | 0.0 | 14:35–18:24 | 1.175 [1.023, 1.327] | 807 | 3.21 | 30.3 | 31.8 | 2.17 | 2.54 | 2.16 | 0.463 | 0.544 |
| W05 | 21 Aug | V1 | 77.0 | 11:12–15:44 | 0.500 [0.447, 0.553] | 910 | 3.25 | 30.2 | 34.5 | 2.00 | 3.47 | 2.29 | 0.144 | 0.218 |
| W06 | 24 Aug | none | 0.0 | 07:57–13:00 | 1.371 [1.180, 1.562] | 872 | 3.29 | 30.3 | 32.8 | 2.42 | 2.57 | 1.93 | 0.533 | 0.711 |
| W07 | 24 Aug | V1 | 77.0 | 13:25–18:01 | 0.615 [0.601, 0.629] | 916 | 3.31 | 35.0 | 35.6 | 2.13 | 3.69 | 3.50 | 0.166 | 0.176 |
| W08 | 25 Aug | V2 | 77.0 | 11:25–17:18 | 0.446 [0.349, 0.543] | 915 | 3.25 | 30.6 | 35.5 | 2.07 | 3.72 | 2.33 | 0.120 | 0.191 |
| W09 | 26 Aug | V2 | 74.1 | 09:47–18:49 | 0.354 [0.318, 0.391] | 733 | 3.19 | 30.1 | 33.2 | 2.34 | 2.75 | 1.93 | 0.129 | 0.184 |
| W10 | 27 Aug | V2 | 74.1 | 09:22–17:04 | 0.234 [0.204, 0.265] | 626 | 3.50 | 30.5 | 30.5 | 2.62 | 1.74 | 1.75 | 0.134 | 0.134 |
| W11 | 28 Aug | V2 | 74.1 | 07:57–16:55 | 0.370 [0.345, 0.395] | 285 | 3.63 | 30.8 | 31.8 | 2.23 | 2.46 | 2.22 | 0.150 | 0.166 |
| W12 | 31 Aug | V2 | 74.1 | 08:45–17:37 | 0.420 [0.328, 0.513] | 940 | 0.00 | 30.3 | 35.0 | 2.34 | 3.30 | 2.00 | 0.127 | 0.210 |
| W13 | 1 Sep | V2 | 74.1 | 08:35–13:35 | 0.336 [0.295, 0.377] | 864 | 0.00 | 32.5 | 34.1 | 2.54 | 2.81 | 2.35 | 0.120 | 0.143 |
| W14 | 2 Sep | V2 | 74.1 | 07:56–16:30 | 0.242 [0.210, 0.274] | 731 | 0.00 | 32.7 | 33.7 | 2.25 | 2.98 | 2.70 | 0.081 | 0.090 |
| W15 | 3 Sep | V2 | 74.1 | 08:08–16:39 | 0.118 [0.102, 0.134] | 152 | 0.00 | 31.6 | 31.9 | 2.60 | 2.12 | 2.04 | 0.055 | 0.058 |

E: negative OLS slope of the 1 min level record. 95 % CI: Newey–West standard error (Bartlett kernel, 120 min bandwidth) with a fixed-b critical value simulated for the slope of a time trend. It describes the measurement precision of one window, not the day-to-day variability of rates (Table 4). In simulated records with the dependence of the real residuals (coverage_check.py) the nominal 95 % interval covered the true rate in a median 0.86 of cases (minimum 0.76); the intervals are therefore somewhat narrow.

Rs: SP-LITE reading (silicon cell, 400–1100 nm) in the water-surface plane, an index of lamp output rather than the radiant load. Tw: reference thermistor. VPD = es(Ta) − ea; Δe = es(Tw) − ea. E/VPD and E/Δe in mm h⁻¹ kPa⁻¹.

Data file: data/processed/window_rates.csv; DS01_rate_estimators.csv.

Table 3. Suppression of each covered–uncovered comparison with measurement 95 % CIs.

| Comparison | Regime | S raw (%) | S VPD-norm. (%) | S Δe-norm. (%) |
| --- | --- | --- | --- | --- |
| 18 vs 17 Aug, V1 | weak | 71.0 [62.6, 78.7] | 65.5 [55.6, 74.8] | 66.2 [56.5, 75.2] |
| 21 vs 20 Aug, V1 | strong | 57.4 [49.6, 63.7] | 68.9 [63.2, 73.4] | 59.9 [52.5, 65.7] |
| 24 Aug, paired, V1 | strong | 55.1 [47.9, 60.8] | 68.8 [63.7, 72.7] | 75.2 [71.2, 78.4] |
| 25 vs 24 Aug, V2 † | strong | 67.5 [58.4, 75.4] | 77.5 [71.2, 83.0] | 73.1 [65.6, 79.6] |

S = 100 (1 − E_cov/E_unc), each rate optionally divided by the mean VPD or Δe of its own window. CIs propagate the measurement interval of both rates (Table 2) by simulation; they ignore day-to-day variability and are not the uncertainty of the cover's efficiency (Table 4).

† Confounded by water temperature: the V1 and V2 water-temperature ranges do not overlap.

Data file: T3_suppression_ci.csv.

Table 4. Window-level model of suppression (primary inference).

|  | Raw | VPD-normalised | Δe-normalised |
| --- | --- | --- | --- |
| S, weak regime [95 % CI] | 75.1 [52.7, 86.9] | 68.7 [51.2, 80.0] | 67.1 [42.7, 81.1] |
| S, strong regime [95 % CI] | 56.3 [26.1, 74.2] | 68.8 [55.1, 78.3] | 68.5 [50.4, 80.0] |
| S weak − S strong, pp [95 % CI] | 18.8 [−9.2, 46.9] | −0.1 [−18.1, 17.9] | −1.4 [−24.5, 21.8] |
| Interaction p (3 d.f.) | 0.12 | 0.99 | 0.86 |
| Equivalence within ±10 pp (90 % CI) | not demonstrated | not demonstrated | not demonstrated |
| Minimum detectable difference, pp | 41 | 25 | 32 |
| with 4 days per cell, pp | 18 | 11 | 14 |
| Pooled S, no interaction [95 % CI] | 65.1 [43.0, 78.7] | 68.8 [61.4, 74.8] | 67.9 [58.1, 75.4] |
| 95 % prediction interval, new pair | 3–87 | 51–80 | 44–82 |
| Between-window SD of ln rate | 0.229 | 0.099 | 0.124 |
| Mixed model, 60 min blocks: S weak / strong | 74.9 / 56.4 | 68.3 / 68.7 | 67.2 / 69.0 |
| Mixed model: interaction p | 0.14 | 0.96 | 0.84 |

OLS of ln(E/X) on cover, regime and their interaction, one observation per window, using the seven V1 and uncovered windows (uncovered weak: 17 and 19 Aug; covered weak: 18 Aug; uncovered strong: 20 and 24 Aug; covered strong: 21 and 24 Aug), with X = 1, VPD or Δe. The residual variance is the day-to-day variability of rates under nominally identical conditions, so these intervals carry the uncertainty that matters for the cover's efficiency. 'Regime' combines fan setting and lamp setting: the strong-regime windows also had a silicon-band irradiance index about five times higher.

Pooled S from the model without interaction (4 d.f.). Minimum detectable difference at α = 0.05 and power 0.8 from the noncentral t (3 d.f.). The ±10 pp equivalence margin was fixed before looking at the result. Mixed model: same fixed effects on 60 min block rates with a random intercept per window.

Data file: T4_window_level_model.csv; DS04_mixed_model_blocks.csv; DS24_mdd_projection.csv.

Table 5. Within-window elasticity γ of the evaporation rate to the driving variable.

| Blocks | Subset | Driver | γ PPML [95 % CI] | p (γ = 1) | γ PPML, E ≥ 0 | γ log-OLS [95 % CI] | Blocks (windows) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60 min | all | VPD | 2.01 [1.32, 2.71] | 0.008 | 1.90 | 2.26 [1.09, 3.42] | 98 (15) |
| 60 min | all | Δe | 1.06 [0.76, 1.35] | 0.70 | 1.05 | 1.34 [0.58, 2.10] | 98 (15) |
| 60 min | uncovered ‡ | VPD | 1.82 [1.02, 2.62] | 0.05 | 1.82 | 1.71 [0.43, 3.00] | 22 (4) |
| 60 min | uncovered ‡ | Δe | 0.88 [0.49, 1.27] | 0.40 | 0.88 | 0.91 [0.15, 1.67] | 22 (4) |
| 60 min | covered | VPD | 2.20 [0.95, 3.45] | 0.06 | 1.97 | 2.41 [0.94, 3.88] | 76 (11) |
| 60 min | covered | Δe | 1.27 [0.97, 1.58] | 0.07 | 1.26 | 1.51 [0.51, 2.51] | 76 (11) |
| 30 min | all | VPD | 2.05 [1.11, 2.98] | 0.03 | 1.70 | 1.97 [0.79, 3.15] | 199 (15) |
| 30 min | all | Δe | 0.99 [0.60, 1.37] | 0.94 | 0.89 | 1.47 [0.65, 2.28] | 199 (15) |
| 30 min | uncovered ‡ | VPD | 2.02 [1.75, 2.29] | 0.001 | 2.02 | 1.85 [0.69, 3.02] | 43 (4) |
| 30 min | uncovered ‡ | Δe | 0.97 [0.53, 1.41] | 0.83 | 0.97 | 1.05 [0.28, 1.81] | 43 (4) |
| 30 min | covered | VPD | 2.07 [0.15, 4.00] | 0.24 | 1.40 | 2.01 [0.50, 3.52] | 156 (11) |
| 30 min | covered | Δe | 1.01 [0.16, 1.86] | 0.98 | 0.79 | 1.65 [0.57, 2.73] | 156 (11) |

E_b = exp(α_w + γ ln X_b) fitted to block rates with a fixed effect per window, so γ is identified only from variation within windows, where cover, plate count, fan and lamp setting are constant. γ = 1 is the proportionality that justifies dividing a rate by X. PPML (Poisson pseudo-maximum likelihood) uses all blocks, including the few negative block rates, which are measurement noise around a small true rate; the 'E ≥ 0' column shows the estimate when they are dropped. CI with cluster-robust SEs by window and t(G − 1). Log-OLS drops blocks with E ≤ 0.

‡ Only four windows: cluster-robust intervals and p-values are unreliable and are descriptive only.

Data file: T5_elasticity.csv.

Table 6. Paired run of 24 August: tests on 30 min block rates.

| Statistic | Value |
| --- | --- |
| Blocks, uncovered (08:00–13:00) / covered (13:30–18:00) | 10 / 9 |
| Median block rate (mm h⁻¹), uncovered / covered | 1.380 / 0.654 |
| Mann–Whitney U; exact p | 84; 6.5 × 10⁻⁴ |
| normal approximation with continuity correction | 0.002 |
| Exact permutation test on the mean difference, p | 2.2 × 10⁻⁴ |
| Welch t (d.f.); p | 5.43 (11.7); 1.6 × 10⁻⁴ |
| Hodges–Lehmann shift, mm h⁻¹ [95 % CI] | 0.696 [0.436, 0.932] |
| Hedges g [exact 95 % CI] | 2.29 [1.12, 3.42] |
| Trend of uncovered block rates in time, Spearman ρ (p) | 0.92 (2.0 × 10⁻⁴) |
| Trend of covered block rates in time, Spearman ρ (p) | 0.17 (0.67) |

All uncovered blocks precede all covered blocks and the uncovered rate rises through the morning, so the blocks are not exchangeable: the tests compare the uncovered morning with the covered afternoon, and their p-values assume no time-of-day effect. The VPD and Δe ranges of the two windows do not overlap.

Data file: T6_paired_24Aug_tests.csv.

Supplementary tables

Table S1. Rate estimators, measurement uncertainty and residual diagnostics.

| Window | Date | Cover | n | E OLS | SE OLS | SE HAC-120 | Fixed-b c.v. | SE AR-sieve | SE HAC Andrews | SE MBB-60 | E GLS-AR(1) | DW | ρ₁ | ACF 30 min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W01 | 17 Aug | none | 487 | 0.3488 | 0.0025 | 0.0124 | 3.29 | 0.0201 | 0.0057 | 0.0117 | 0.3478 | 1.08 | 0.45 | 0.27 |
| W02 | 18 Aug | V1 | 421 | 0.1013 | 0.0021 | 0.0073 | 3.45 | 0.0107 | 0.0033 | 0.0085 | 0.1019 | 1.34 | 0.32 | 0.26 |
| W03 | 19 Aug | none | 322 | 0.4763 | 0.0038 | 0.0091 | 3.78 | 0.0070 | 0.0052 | 0.0086 | 0.4765 | 1.55 | 0.22 | 0.08 |
| W04 | 20 Aug | none | 230 | 1.1746 | 0.0083 | 0.0340 | 4.46 | 0.0265 | 0.0230 | 0.0368 | 1.1784 | 1.11 | 0.44 | 0.10 |
| W05 | 21 Aug | V1 | 273 | 0.4998 | 0.0040 | 0.0127 | 4.16 | 0.0093 | 0.0074 | 0.0131 | 0.5030 | 1.31 | 0.30 | 0.21 |
| W06 | 24 Aug | none | 304 | 1.3708 | 0.0077 | 0.0473 | 4.03 | 0.0401 | 0.0359 | 0.0474 | 1.3825 | 0.43 | 0.77 | 0.34 |
| W07 | 24 Aug | V1 | 277 | 0.6151 | 0.0034 | 0.0034 | 4.04 | 0.0070 | 0.0045 | 0.0060 | 0.6157 | 1.59 | 0.19 | −0.12 |
| W08 | 25 Aug | V2 | 354 | 0.4460 | 0.0042 | 0.0265 | 3.64 | 0.0278 | 0.0185 | 0.0251 | 0.4507 | 0.46 | 0.76 | 0.38 |
| W09 | 26 Aug | V2 | 543 | 0.3541 | 0.0017 | 0.0114 | 3.22 | 0.0096 | 0.0062 | 0.0093 | 0.3553 | 0.80 | 0.59 | 0.32 |
| W10 | 27 Aug | V2 | 463 | 0.2345 | 0.0019 | 0.0090 | 3.34 | 0.0059 | 0.0061 | 0.0079 | 0.2359 | 0.94 | 0.50 | 0.11 |
| W11 | 28 Aug | V2 | 539 | 0.3698 | 0.0015 | 0.0078 | 3.26 | 0.0054 | 0.0037 | 0.0061 | 0.3703 | 1.24 | 0.37 | 0.19 |
| W12 | 31 Aug | V2 | 533 | 0.4205 | 0.0032 | 0.0283 | 3.26 | 0.0319 | 0.0230 | 0.0222 | 0.4330 | 0.14 | 0.92 | 0.67 |
| W13 | 1 Sep | V2 | 301 | 0.3359 | 0.0031 | 0.0104 | 3.91 | 0.0107 | 0.0066 | 0.0115 | 0.3374 | 1.01 | 0.48 | 0.03 |
| W14 | 2 Sep | V2 | 515 | 0.2420 | 0.0016 | 0.0098 | 3.26 | 0.0087 | 0.0068 | 0.0079 | 0.2429 | 0.43 | 0.78 | 0.15 |
| W15 | 3 Sep | V2 | 512 | 0.1176 | 0.0012 | 0.0049 | 3.28 | 0.0071 | 0.0039 | 0.0050 | 0.1180 | 0.78 | 0.61 | 0.10 |

SE in mm h⁻¹. OLS assumes independent residuals. HAC-120: Newey–West, Bartlett kernel, 120 min bandwidth (adopted), with its fixed-b critical value simulated for a trend slope. AR-sieve: bootstrap from an AR(p) fitted to the residuals. HAC Andrews: AR(1) plug-in bandwidth, too short for these residuals. MBB-60: moving-block bootstrap, 60 min blocks. GLS-AR(1): feasible GLS with AR(1) errors.

Median coverage of nominal 95 % intervals in simulated records (DS00_ci_coverage.csv): OLS 0.30, HAC Andrews 0.55, HAC-60 normal 0.71, MBB-60 0.76, AR-sieve 0.81, HAC-60 fixed-b 0.84, HAC-120 fixed-b 0.86. The simulated noise comes from AR models fitted to the residuals, which understate persistence, so all of these coverages are, if anything, optimistic.

Data file: DS01_rate_estimators.csv; DS00_ci_coverage.csv.

Table S2. Balance of each comparison: what else differed between the covered and uncovered windows.

| Comparison | Time unc. / cov. | Rs unc. / cov. (W m⁻²) | Tw unc. / cov. (°C) | Ta unc. / cov. (°C) | Ta rise unc. / cov. (K) | ea unc. / cov. (kPa) | VPD unc. / cov. (kPa) | Δe unc. / cov. (kPa) | Share of ΔVPD from es(Ta) (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 vs 17 Aug | 09:20–17:26 / 11:20–18:20 ‡ | 106 / 144 | 26.8 / 27.4 | 30.7 / 30.5 | −0.5 / 0.2 | 2.24 / 2.55 | 2.16 / 1.82 | 1.28 / 1.10 | 11 |
| 21 vs 20 Aug | 14:35–18:24 / 11:12–15:44 ‡ | 807 / 910 | 30.3 / 30.2 | 31.8 / 34.5 | 1.5 / 3.4 | 2.17 / 2.00 | 2.54 / 3.47 | 2.16 / 2.29 | 82 |
| 24 Aug, paired | 07:57–13:00 / 13:25–18:01 | 872 / 916 | 30.3 / 35.0 | 32.8 / 35.6 | 3.9 / 1.1 | 2.42 / 2.13 | 2.57 / 3.69 | 1.93 / 3.50 | 74 |
| 25 vs 24 Aug | 07:57–13:00 / 11:25–17:18 ‡ | 872 / 915 | 30.3 / 30.6 | 32.8 / 35.5 | 3.9 / 2.6 | 2.42 / 2.07 | 2.57 / 3.72 | 1.93 / 2.33 | 69 |

‡ Windows on different days. Ta rise: increase of the HMP45C air temperature over the window (linear trend). Until 10 Sep the HMP45C sat at the top of the rig, near the lamps. In C2–C4 most of the VPD increase comes from es(Ta), i.e. from the warmer air measured at the sensor, while ea fell by only 8–14 %.

Data file: DS23_comparison_balance.csv.

Table S3. Sensitivity of suppression (%) to analysis choices and instrument uncertainty.

| Sensitivity analysis | Normaliser | C1 18/17 Aug | C2 21/20 Aug | C3 24 Aug | C4 25/24 Aug |
| --- | --- | --- | --- | --- | --- |
| Window bounds, 256 variants: min–max | Raw | 68.5–72.9 | 53.0–61.3 | 53.0–58.4 | 63.3–71.2 |
| Window bounds, 256 variants: min–max | VPD-normalised | 63.1–68.3 | 67.4–70.5 | 67.9–69.7 | 75.7–78.9 |
| Window bounds, 256 variants: min–max | Δe-normalised | 63.8–67.4 | 58.9–61.2 | 74.8–76.6 | 71.5–74.8 |
| Δe from thermistor 2 | Δe | 67.8 | 50.6 | 62.6 | 72.2 |
| Δe from thermistor 3 | Δe | 66.1 | 59.3 | 75.7 | 73.7 |
| Δe from mean of three | Δe | 66.7 | 56.4 | 71.2 | 72.9 |
| HMP45C RH ± 3 %, Ta ± 0.3 °C, all windows | VPD | 65.1–65.9 | 68.6–69.2 | 68.4–69.1 | 77.2–77.8 |
|  | Δe | 65.1–66.9 | 59.5–60.2 | 74.4–76.3 | 72.8–73.4 |
| RH offset ± 3 % in strong-fan windows only | VPD | 65.5–65.5 | 68.6–69.2 | 68.4–69.1 | 77.3–77.8 |
|  | Δe | 66.2–66.2 | 59.6–60.1 | 74.6–76.0 | 73.0–73.2 |
| Transducer thermal coefficient c = 0.1 mm K⁻¹ | Raw | 75.1 | 58.8 | 52.9 | 69.7 |
|  | VPD-normalised | 70.5 | 69.9 | 67.2 | 79.1 |
| Transducer thermal coefficient c = 0.2 mm K⁻¹ | Raw | 79.5 | 60.5 | 50.1 | 72.6 |
|  | VPD-normalised | 75.6 | 71.1 | 65.2 | 81.1 |
| Transducer thermal coefficient c = 0.3 mm K⁻¹ | Raw | 83.9 | 62.7 | 46.5 | 76.3 |
|  | VPD-normalised | 80.9 | 72.7 | 62.7 | 83.6 |

Window bounds: start delayed and end advanced by 0, 15, 30 or 45 min independently in both windows. Thermistor positions before 10 Sep are not documented; thermistor 2 behaves as a near-surface probe, 0.5–11 K warmer than the others.

Thermal coefficient: rates corrected by c·dTw/dt (probe 1). c is not identified by the data (Table S7), so these rows show what a coefficient of that size would do, not a correction. Effect on the window-level model (DS22): pooled VPD-normalised S 68.8 %, 69.8 %, 71.1 %, 72.7 % for c = 0, 0.1, 0.2, 0.3 mm K⁻¹; interaction p (VPD) 0.99, 0.59, 0.36, 0.30; interaction p (raw) 0.12, 0.07, 0.05, 0.06.

Data file: DS12_window_bounds.csv; DS15_probe_choice.csv; DS16_sensor_accuracy.csv; DS22_thermal_drift_sensitivity.csv.

Table S4. Suppression (%) for every covered–uncovered V1 pair within the same advective regime.

| Covered / uncovered | Regime | S raw | S VPD-norm. | S Δe-norm. |
| --- | --- | --- | --- | --- |
| 18 Aug V1 / 17 Aug | weak | 71.0 [62.6, 78.7] | 65.5 [55.6, 74.8] | 66.2 [56.5, 75.2] |
| 18 Aug V1 / 19 Aug | weak | 78.7 [73.1, 84.1] | 71.6 [64.1, 78.7] | 68.0 [59.5, 76.0] |
| 21 Aug V1 / 20 Aug | strong | 57.4 [49.6, 63.7] | 68.9 [63.2, 73.4] | 59.9 [52.5, 65.7] |
| 21 Aug V1 / 24 Aug | strong | 63.5 [56.4, 69.4] | 73.0 [67.7, 77.3] | 69.3 [63.3, 74.2] |
| 24 Aug V1 / 20 Aug | strong | 47.6 [39.9, 53.4] | 64.0 [58.7, 68.0] | 67.6 [62.8, 71.2] |
| 24 Aug V1 / 24 Aug | strong | 55.1 [47.9, 60.8] | 68.8 [63.7, 72.7] | 75.2 [71.2, 78.4] |

Measurement 95 % CIs. The window-level model of Table 4 uses all of these references at once instead of one hand-picked pair per regime.

Data file: DS14_reference_choice.csv.

Table S5. Micrometer and level-transducer rates over common intervals (mm h⁻¹).

| Date | Interval | State | Readings | Duration (h) | dTw/dt (K h⁻¹) | Expansion (mm) | E micrometer | E micrometer, mass-eq. | E transducer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17 Aug | 09:21–17:21 | uncovered | 10 | 8.0 | 0.04 | 0.04 | 0.374 | 0.376 | 0.346 |
| 18 Aug | 11:04–17:08 | V1 | 4 | 6.1 | 0.18 | 0.07 | 0.076 | 0.087 | 0.104 |
| 19 Aug | 10:12–16:11 | uncovered | 5 | 6.0 | 0.49 | 0.18 | 0.294 | 0.324 | 0.443 |
| 20 Aug | 17:51–18:18 | uncovered | 2 | 0.5 | 0.81 | 0.03 | 2.071 | 2.131 | 1.394 |
| 21 Aug | 11:12–15:40 | V1 | 8 | 4.5 | 0.81 | 0.26 | 0.362 | 0.420 | 0.495 |
| 26 Aug | 10:17–18:49 | V2 | 2 | 8.5 | 0.54 | 0.34 | 0.373 | 0.412 | 0.343 |
| 27 Aug | 09:50–17:04 | V2 | 2 | 7.2 | 0.17 | 0.09 | 0.180 | 0.192 | 0.235 |
| 28 Aug | 08:00–16:53 | V2 | 3 | 8.9 | 0.40 | 0.26 | 0.308 | 0.338 | 0.352 |
| 31 Aug | 09:20–17:40 | V2 | 2 | 8.3 | 1.32 | 0.84 | 0.257 | 0.358 | 0.420 |
| 1 Sep | 08:14–13:38 | V2 | 2 | 5.4 | 1.13 | 0.46 | 0.100 | 0.185 | 0.409 |
| 2 Sep | 07:40–17:20 | V2 | 2 | 9.7 | 0.50 | 0.35 | 0.154 | 0.191 | 0.245 |

Rates by regression over the micrometer reading times; the transducer value at each time is the mean of ±5 min of 1 min records. The micrometer reads surface height, which rises as the water column warms; the transducer reads hydrostatic pressure, i.e. mass. Mass-equivalent: height corrected for thermal expansion with the mean water temperature, assuming the water in the stilling well expands like the pan; the as-read value is the other bound. The 20 Aug interval (27 min, 2 readings) is excluded from the agreement statistics.

Data file: DS17_micrometer_intervals.csv.

Table S6. Agreement between micrometer and transducer rates.

| Set | Micrometer | n | Pearson r | Lin CCC [95 % CI] | Bias (mm h⁻¹) | Limits of agreement | Geometric mean ratio [95 % CI] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| intervals ≥ 2 h | as read | 10 | 0.60 | 0.44 [−0.05, 0.83] | 0.092 | −0.109 to 0.292 | 0.69 [0.51, 0.94] |
| intervals ≥ 2 h | mass-eq. | 10 | 0.76 | 0.68 [0.05, 0.93] | 0.051 | −0.108 to 0.209 | 0.83 [0.69, 1.00] |
| all intervals incl. 27 min on 20 Aug | as read | 11 | 0.97 | 0.85 [0.06, 0.88] | 0.022 | −0.471 to 0.514 | 0.74 [0.55, 1.01] |
| 17–21 Aug (does not reproduce the manuscript) | as read | 5 | 0.98 | 0.86 [−0.21, 0.91] | −0.079 | −0.750 to 0.592 | 0.89 [0.58, 1.36] |

Lin's concordance (1/n moments) with a case-resampling bootstrap CI; it measures agreement with the identity line, which Pearson's r does not. Bias and limits of agreement (Bland–Altman): transducer minus micrometer. Ratio: micrometer / transducer. The last two rows are dominated by the 27 min interval of 20 Aug.

Data file: DS18_micrometer_agreement.csv.

Table S7. Apparent thermal coefficient of the level transducer.

| Source | Variant | n | c (mm K⁻¹) [95 % CI] | p | Leave-one-out range |
| --- | --- | --- | --- | --- | --- |
| Micrometer, day intervals ≥ 2 h | mass-equivalent, mean of probes | 10 | 0.11 [−0.02, 0.24] | 0.08 | 0.05 to 0.18 |
| Micrometer, day intervals ≥ 2 h | as read (no expansion in well), probe 1 | 10 | 0.21 [−0.10, 0.53] | 0.15 | 0.17 to 0.26 |
| Micrometer, day intervals ≥ 2 h | as read (no expansion in well), mean of probes | 10 | 0.19 [0.06, 0.32] | 0.009 | 0.13 to 0.26 |
| Night-time level, E_app = k Δe + c dT/dt | V2, 26 plates (26 Aug–2 Sep); probe 1 | 6 | 0.43 [0.11, 0.75] | 0.02 | 0.36 to 0.55 |
| Night-time level, E_app = k Δe + c dT/dt | V2, 26 plates (26 Aug–2 Sep); mean of probes | 6 | 0.15 [−0.15, 0.45] | 0.24 | 0.09 to 0.32 |
| Night-time level, E_app = k Δe + c dT/dt | V2, all; probe 1 | 7 | −0.12 [−0.61, 0.37] | 0.56 | −0.20 to 0.43 |
| Night-time level, E_app = k Δe + c dT/dt | V2, all; mean of probes | 7 | 0.01 [−0.42, 0.44] | 0.94 | −0.07 to 0.15 |
| Night-time level, E_app = k Δe + c dT/dt | all covered; probe 1 | 10 | −0.16 [−0.44, 0.12] | 0.23 | −0.20 to −0.04 |
| Night-time level, E_app = k Δe + c dT/dt | all covered; mean of probes | 10 | −0.02 [−0.31, 0.27] | 0.86 | −0.06 to 0.06 |

c > 0 means the reading falls when the water warms (apparent extra evaporation) and rises when it cools. On the night of 1–2 Sep the level rose by about 0.2 mm while the covered pan could only lose water. Point estimates range from −0.16 to 0.43 mm K⁻¹ (leave-one-out −0.20 to 0.55) depending on the nights, temperature and expansion assumption used: c is not identified by these data. A bench test at constant water mass is needed.

Data file: DS20_night_thermal_fit.csv; DS21_micrometer_drift_fit.csv.

Table S8. Dark periods (lamps and fan off) used for the night-time check.

| Start | Hours | Cover | E_app (mm h⁻¹) | dT/dt probe 1 (K h⁻¹) | Δe (kPa) | Largest 3 h change (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| 17 Aug 18:41 | 14.7 | uncovered | 0.022 | −0.01 | 1.05 | 0.00 |
| 18 Aug 19:07 | 14.8 | V1 | 0.028 | −0.04 | 1.27 | 0.00 |
| 19 Aug 16:42 | 17.4 | uncovered | 0.083 | −0.18 | 1.41 | −0.04 |
| 20 Aug 19:25 | 12.3 | uncovered | 0.103 | −0.35 | 1.90 | −0.23 |
| 21 Aug 16:45 | 63.0 | V1 | 0.047 | −0.05 | 1.29 | 0.00 |
| 24 Aug 19:02 | 15.9 | V1 | 0.094 | −0.37 | 2.07 | 0.00 |
| 25 Aug 18:19 | 13.3 | V2 | 0.086 | −0.32 | 1.93 | −0.04 |
| 26 Aug 19:50 | 11.7 | V2 | 0.031 | −0.13 | 2.19 | 0.01 |
| 27 Aug 19:23 | 12.4 | V2 | 0.039 | −0.12 | 1.87 | 0.00 |
| 28 Aug 17:56 | 62.3 | V2 | 0.044 | −0.06 | 1.33 | 0.00 |
| 31 Aug 18:38 | 13.4 | V2 | 0.043 | −0.24 | 2.78 | 0.05 |
| 1 Sep 17:21 | 14.4 | V2 | −0.003 | −0.29 | 2.74 | 0.20 |
| 2 Sep 17:31 | 14.4 | V2 | 0.008 | −0.25 | 2.57 | 0.00 |

First 60 min after lamp switch-off and the last 10 min removed; periods cut before any level step (refill or plate operation). Largest 3 h change: maximum increase of the 60 min running mean over 3 h (negative when the level never rose).

Data file: DS19_night_periods.csv.

Table S9. Advection and plate-geometry contrasts (ratio of rates).

| Contrast | Normaliser | Ratio | Measurement 95 % CI | Between-day 95 % CI |
| --- | --- | --- | --- | --- |
| covered V2-26, 26 Aug (u 3.19) / 2 Sep (u 0), Rs 733/731 | Raw | 1.46 | 1.24–1.73 | 0.60–3.59 |
| uncovered strong (20, 24 Aug) / weak (17, 19 Aug); Rs ratio 5.3 | Raw | 3.09 | – | 1.63–5.82 |
| cover V2 (25 Aug) / cover V1 (24 Aug), strong advection | Raw | 0.73 | 0.56–0.88 | 0.30–1.78 |
| covered V2-26, 26 Aug (u 3.19) / 2 Sep (u 0), Rs 733/731 | VPD-normalised | 1.59 | 1.35–1.88 | 1.08–2.34 |
| uncovered strong (20, 24 Aug) / weak (17, 19 Aug); Rs ratio 5.3 | VPD-normalised | 2.79 | – | 2.12–3.67 |
| cover V2 (25 Aug) / cover V1 (24 Aug), strong advection | VPD-normalised | 0.72 | 0.55–0.87 | 0.49–1.06 |
| covered V2-26, 26 Aug (u 3.19) / 2 Sep (u 0), Rs 733/731 | Δe-normalised | 2.05 | 1.74–2.43 | 1.26–3.34 |
| uncovered strong (20, 24 Aug) / weak (17, 19 Aug); Rs ratio 5.3 | Δe-normalised | 2.24 | – | 1.59–3.16 |
| cover V2 (25 Aug) / cover V1 (24 Aug), strong advection | Δe-normalised | 1.09 | 0.83–1.31 | 0.67–1.77 |

Between-day CI: the between-window SD of ln rate from Table 4 (t, 4 d.f.). The uncovered strong/weak ratio compares windows whose silicon-band irradiance differed about fivefold, so it cannot be attributed to advection. The covered pair of 26 Aug and 2 Sep is the only contrast at a matched irradiance index. V2/V1 compares two different days and cannot be resolved once day-to-day variability is included.

Data file: DS07_advection_contrasts.csv.

Table S10. Covered rate (mm h⁻¹) of cover V2 with 26 plates against window-mean forcing.

| Series | x | n | Slope [95 % CI] | R² | p | Spearman ρ (exact p) |
| --- | --- | --- | --- | --- | --- | --- |
| calm (u = 0), V2 26 plates | Rs (per 100 W m⁻²) | 4 | 0.034 [−0.004, 0.073] | 0.88 | 0.06 | 1.00 (0.08) |
| calm (u = 0), V2 26 plates | VPD (per kPa) | 4 | 0.235 [−0.114, 0.584] | 0.81 | 0.10 | 0.80 (0.33) |
| calm (u = 0), V2 26 plates | Δe (per kPa) | 4 | −0.040 [−1.258, 1.179] | 0.01 | 0.90 | −0.40 (0.75) |
| calm (u = 0), V2 26 plates | Ta (per K) | 4 | 0.096 [0.041, 0.151] | 0.97 | 0.02 | 1.00 (0.08) |
| strong advection, V2 26 plates | Rs (per 100 W m⁻²) | 3 | −0.012 [−0.383, 0.358] | 0.15 | 0.75 | −0.50 (1.00) |
| strong advection, V2 26 plates | VPD (per kPa) | 3 | 0.133 [−0.552, 0.817] | 0.86 | 0.25 | 0.50 (1.00) |
| strong advection, V2 26 plates | Δe (per kPa) | 3 | 0.263 [−1.869, 2.394] | 0.71 | 0.36 | 1.00 (0.33) |
| strong advection, V2 26 plates | Ta (per K) | 3 | 0.043 [−0.385, 0.472] | 0.62 | 0.42 | 0.50 (1.00) |

With three or four windows no slope is significant at the 5 % level except Ta in the calm series, and the smallest attainable exact two-sided Spearman p is 0.33 (n = 3) or 0.083 (n = 4). In the calm series Rs, Ta and VPD are collinear (r ≥ 0.94).

Data file: DS08_covered_rate_regressions.csv.

Table S11. Latent heat flux, heat storage and the silicon-pyranometer readings (W m⁻²).

| Window | Date | Cover | Rs SP-LITE | Rs LI200X | LE [95 % CI] | G probe 1 | LE/Rs (%) | (LE + G)/Rs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W01 | 17 Aug | none | 106 | 114 | 236 [209, 264] | 11 | 224 | 234 |
| W02 | 18 Aug | V1 | 144 | 162 | 69 [52, 85] | 40 | 47 | 75 |
| W03 | 19 Aug | none | 213 | 248 | 322 [297, 344] | 148 | 151 | 221 |
| W04 | 20 Aug | none | 807 | 776 | 793 [689, 889] | 275 | 98 | 132 |
| W05 | 21 Aug | V1 | 910 | 1043 | 337 [302, 373] | 162 | 37 | 55 |
| W06 | 24 Aug | none | 872 | 976 | 925 [794, 1056] | 358 | 106 | 147 |
| W07 | 24 Aug | V1 | 916 | 1042 | 413 [404, 422] | 90 | 45 | 55 |
| W08 | 25 Aug | V2 | 915 | 1038 | 301 [230, 365] | 181 | 33 | 53 |
| W09 | 26 Aug | V2 | 733 | 842 | 239 [215, 264] | 122 | 33 | 49 |
| W10 | 27 Aug | V2 | 626 | 689 | 158 [137, 178] | 30 | 25 | 30 |
| W11 | 28 Aug | V2 | 285 | 310 | 249 [232, 266] | 87 | 88 | 118 |
| W12 | 31 Aug | V2 | 940 | 1091 | 284 [220, 350] | 213 | 30 | 53 |
| W13 | 1 Sep | V2 | 864 | 1008 | 226 [199, 252] | 101 | 26 | 38 |
| W14 | 2 Sep | V2 | 731 | 807 | 163 [141, 184] | 83 | 22 | 34 |
| W15 | 3 Sep | V2 | 152 | 184 | 79 [69, 90] | 46 | 52 | 82 |

Both pyranometers are silicon cells (400–1100 nm) calibrated to sunlight; most of the output of the infrared lamps lies beyond 1100 nm, so Rs underestimates the radiant load by a factor that grows as the lamps are dimmed. LE/Rs is therefore not an energy partition. G = ρ c h dTw/dt from probe 1.

Data file: DS11_energy_terms.csv.

Table S12. Heterogeneity of the three V1 comparisons using measurement variances only.

| Variance | Normaliser | Q | p | I² (%) | Pooled S [95 % CI] |
| --- | --- | --- | --- | --- | --- |
| HAC-120 (adopted) | Raw | 7.6 | 0.02 | 74 | 60.6 [31.9, 77.3] |
| HAC-120 (adopted) | VPD-normalised | 0.4 | 0.81 | 0 | 68.4 [60.6, 74.7] |
| HAC-120 (adopted) | Δe-normalised | 19.3 | 6.6 × 10⁻⁵ | 90 | 67.9 [33.3, 84.5] |
| AR-sieve | Raw | 12.6 | 0.002 | 84 | 59.3 [34.5, 74.7] |
| AR-sieve | VPD-normalised | 0.7 | 0.71 | 0 | 68.7 [65.8, 71.4] |
| AR-sieve | Δe-normalised | 127.3 | 2.3 × 10⁻²⁸ | 98 | 67.8 [26.4, 85.9] |

DerSimonian–Laird random effects on ln(E_cov/E_unc) with the modified Hartung–Knapp interval. With k = 3 and variances that omit day-to-day variability, Q depends on the variance estimator and is shown for completeness only; Table 4 gives the inference.

Data file: DS06_heterogeneity_measurement.csv.